====================================================================

Advice to Released Library Revisions

====================================================================

PTP_Drives_Festo_MB FB | Communication: ModbusTCP Simple CPU Communication

------------------------------------------------------------------------------------------------------------------------

Version                           Date of Release

V1.2                           	15.09.2022

V1.3			20.08.2023

------------------------------------------------------------------------------------------------------------------------

Changes in Library:

V1.20 	Development Completed

Note:

The Block from the outside looks the same, and the App Note also has not changed. But there was an internal Bugfix that we fixed, so there is a new version.

BUG:

An issue when dropping the CancelTraversing or IntermediateStop bit to stop the system.  IF they don’t follow the proper sequencing of signals then the function block locks itself up and cannot perform another function.  I have examples of what we saw:

• Good Sequencing – If we follow this procedure, it works every time

o After AxisAckSetpoint goes true, use this signal to drop ExecuteMode to make sure new setpoint was taken.

o If dropping CancelTraversing or IntermediateStop then make sure ExecuteMode is off.

o Drop Enable Bit

o Turn back on Cancel Traversing or IntermediateStop, but make sure both signals are true

o Turn enable back on, when we get AxisEnabled goes true then can send new setpoint and execute

• Bad Sequencing/Lock – up

o If they drop CancelTraversing or Intermediate Stop to stop axis.  If they then just bring cancel traversing signal back to true, nothing happens.

o If they then toggle the enable bit, Automation suite will say it is enabled, but no feedback in the drive, or no AxisEnabled feedback.  Sometimes this will come on after 10 seconds, sometimes not, regardless the drive will no longer take a new command or move.

o Seems the function block is getting locked up

o No error or warning is present in the drive or AOI that indicates this lock up state which is a problem.

• Work-Around

o If we got into that lock up scenario, if I change to Mode 0, to induce an error in the AOI, customer can clear the error and the lock up, and then they can follow proper sequencing and off and running again.

V1.3	Up-dation: PTP_Drives_Festo_MB version V1.3

Developed PTP_Drives_Festo_Parameter Function Block V1.0

Documentation: Added Chapter 3.3 “Simple CPU Communication Settings”

Chapter 8 “Sample Code”, Chapter 9 “Extended Process Data”, Chapter 10 “Record Table”, Chapter 11 “Sample Code description” & Chapter 12” Additional Information”,

The following lines of code are bits that start movement of the axis with pre-defined setpoints (Position/Accel/Decel/etc)

Within the scan, these bits were prior to pre-defined setpoints (Position/Accel/Decel/etc) and therefore could start movement with incorrect setpoints.

Within the scan, these bits were all moved after such setpoints to the end of the raw data outputs.

DRV_Control[0].6 := stat_CycWrite.STW1.ActivateTraversing;

DRV_Control[0].11 := stat_CycWrite.STW1.StartHoming;

DRV_Control[2].1 := stat_CycWrite.EPosSTW2.SetReferencePoint;

DRV_Control[3].8 := stat_CycWrite.STW2.TravelToFixStop;