====================================================================

Advice to Released Library Revisions

====================================================================

CPX-AP-Festo-Lib | Communication: PROFINET

------------------------------------------------------------------------------------------------------------------------

Library Created Software Version: SIMATIC Manager (V5.6)

------------------------------------------------------------------------------------------------------------------------

Version			Date of Release			Developer

V1.0			(10-2023)			DDHN

V1.1			(03-2024)			DDHN

V1.2			(08-2026)			BBSW

------------------------------------------------------------------------------------------------------------------------

Changes in Library:

V1.0 - Release Version

V1.1 - Update Version

CPXAP_Parameter (V1.1)

- Fixed Area Length Error on RDREC/WRREC

- Fixed WRREC LEN from fixed 100 to actual size

- Fixed reset logic, error path flag clearing

- Fixed iBlockType INT changed to WORD - Removed unused variables.

V1.2 - Update Version

CPXAP_Parameter (V1.2)

- Fixed CPU error 16#2523 (Area Length Error at Instance DB offset 126).

- Root cause: STRING without explicit length in dDataType STRUCT defaults to 256 bytes in S7-300, inflating DB layout beyond CPU boundary.

- Removed dDataType STRUCT (unused)

- Removed iMaxReadPayload, replaced with inline constants

- NOTE: Delete old Instance DB, recompile and regenerate DB after this update